PRINTABLE DIAMOND KITE TEMPLATE

Cut-out pattern + build instructions  ·  Finished size approx. 50 × 38 cm when scaled 200%

You will need

2 wooden dowels (50 cm & 38 cm) · paper or plastic sheet · strong string · tape/glue · ribbon for tail · flying line & handle

Steps

Print at 200% (or tile on 4 sheets) and cut along the dashed line.

Tie the dowels in a cross where the red lines meet; notch the ends.

Run string around the four notched ends to form the frame.

Fold the hem over the string and tape it down.

Attach a bridle from the top to bottom of the spine, tie the flying line 1/3 from the top.

Add a 2–3 m ribbon tail at the bottom point for stability.